Introduction

The present report represents a further development of the network project created in Phase 1. In this phase, the configuration of the network is finished, the connectivity of the two campuses is established, and the necessary tests are conducted for successful interaction of the devices. The report contains the procedure of implementation, configuration commands, and the results of testing.

- List all devices used in the project

| Devices | Type |
| --- | --- |
| PC0 | PC-PT |
| PC1 | PC-PT |
| PC2 | PC-PT |
| PC3 | PC-PT |
| PC4 | PC-PT |
| PC5 | PC-PT |
| PC6 | PC-PT |
| PC7 | PC-PT |
| Laptop0 | Laptop-PT |
| Laptop1 | Laptop-PT |
| Laptop2 | Laptop-PT |
| Laptop3 | Laptop-PT |
| Switch0 | 2950-24 |
| Switch1 | 2950-24 |
| Switch2 | 2950-24 |
| Switch3 | 2950-24 |
| Router0 | 2811 |
| Router1 | 2811 |

- commands used to name the devices

Router 0

Router>ena

Router>enable

Router#conf

Router#configure t

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hos

Router(config)#hostname n

Router(config)#hostname na

Router(config)#hostname R1

R1(config)#

R1(config)#

R1(config)#

Router 1

Router>

Router>en

Router>enable

Router#conf

Router#configure t

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#hos

Router(config)#hostname R2

R2(config)#

R2(config)#

- Cable Types Used

| From | Port | To | Port | Cable |
| --- | --- | --- | --- | --- |
| PC0 | Fa0 | Switch0 | Fa0/1 | Straight-through |
| PC1 | Fa0 | Switch0 | Fa0/2 | Straight-through |
| Laptop0 | Fa0 | Switch0 | Fa0/3 | Straight-through |
| Switch0 | Fa0/24 | Router0 | Fa0/0 | Straight-through |
| PC2 | Fa0 | Switch1 | Fa0/1 | Straight-through |
| PC3 | Fa0 | Switch1 | Fa0/2 | Straight-through |
| Laptop1 | Fa0 | Switch1 | Fa0/3 | Straight-through |
| Switch1 | Fa0/24 | Router0 | Fa0/1 | Straight-through |
| PC4 | Fa0 | Switch2 | Fa0/1 | Straight-through |
| PC5 | Fa0 | Switch2 | Fa0/2 | Straight-through |
| Laptop | Fa0 | Switch2 | Fa0/3 | Straight-through |
| Switch2 | Fa0/24 | Router1 | Fa0/0 | Straight-through |
| PC6 | Fa0 | Switch3 | Fa0/1 | Straight-through |
| PC7 | Fa0 | Switch3 | Fa0/2 | Straight-through |
| Laptop3 | Fa0 | Switch3 | Fa0/3 | Straight-through |
| Switch3 | Fa0/24 | Router1 | Fa0/1 | Straight-through |
| Router0 | Se0/3/0 | Router1 | Se03/0 | Serial-DTE DCE |

- Straight-Through = Connect different devices (PC ↔ Switch, Router ↔ Switch).

- Serial DCE/DTE = Connect two routers over a WAN serial link, where one side must provide the clock signal.

- commands used to configure the router LAN interfaces

- Router 0

R1#

R1#en

R1#enable

R1#con

R1#conf

R1#configure t

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#

R1(config)#int

R1(config)#interface f

R1(config)#interface fastEthernet 0/0

R1(config-if)#ip address 172.16.10.1 255.255.255.0

R1(config-if)#

R1#

R1#configure terminal

R1(config)#interface FastEthernet0/0

R1(config-if)#

R1(config-if)#exit

R1(config)#interface FastEthernet0/1

R1(config-if)#shutdown

R1(config-if)#

R1#en

R1#enable

R1#conf

R1#configure t

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#inf

R1(config)#int

R1(config)#interface f

R1(config)#interface fastEthernet 0/1

R1(config-if)#ip ad

R1(config-if)#ip address 172.16.20.1 255.255.255.0

R1(config-if)#

- Router 1

R2#en

R2#enable

R2#conf

R2#configure t

R2#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R2(config)#inf

R2(config)#int

R2(config)#interface f

R2(config)#interface fastEthernet 0/0

R2(config-if)#ip ad

R2(config-if)#ip address 172.16.30.1 255.255.255.0

R2(config-if)#ex

R2(config-if)#exit

R2(config)#inf

R2(config)#int

R2(config)#interface f

R2(config)#interface fastEthernet 0/1

R2(config-if)#ip ad

R2(config-if)#ip address 172.16.40.1 255.255.255.0

R2(config-if)#

- WAN connection

- commands used to configure RIP version 2

- Router 0 RIP

R1>en

R1>enable

R1#conf

R1#configure t

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#ro

R1(config)#router r

R1(config)#router rip

R1(config-router)#v

R1(config-router)#version 2

R1(config-router)#no

R1(config-router)#no at

R1(config-router)#no auto

R1(config-router)#no auto-summary

R1(config-router)#ex

R1(config-route

R1(config)#

R1(config)#

R1(config)#ro

R1(config)#router ri

R1(config)#router rip

R1(config-router)#ne

R1(config-router)#network 10.0.0.0

R1(config-router)#ne

R1(config-router)#network 172.16.0.0

R1(config-router)#

- Router1 RIP

R2>en

R2>enable

R2#conf

R2#configure t

R2#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R2(config)#ro

R2(config)#router r

R2(config)#router rip

R2(config-router)#no

R2(config-router)#no auto

R2(config-router)#no auto-summary

R2(config-router)#

R2(config-router)#ne

R2(config-router)#network 10.0.0.0

R2(config-router)#ne

R2(config-router)#network 172.16.0.0

R2(config-router)#ex

R2(config-router)#exit

R2(config)#

- ping tests between a device

- Conclusion Phase 2

Phase 2 was successfully completed and tested. The RIP version 2 was configured and the auto summary turned off and the connectivity between the routers and networks checked .

- Osama: Designed the network topology, listed all routers used in the project, and configured the commands to name the routers.

- Qasim: Assigned IP addresses to all PCs and laptops and listed the cable types used throughout the project.

- Saud: Provided IP addresses, subnet masks, and serial numbers, fixed network issues during testing, configured RIP Version 2, and tested connectivity between devices in both directions.

Project Links :

GitHub : https://github.com/SaudAldossary0/Porject-360